FrSky 2.4GHz Radio Control System

Instruction Manual for TF Receivers

- TF Receivers

Read the instruction manual and set up the devices as described below.

1.1 Compatibility:

Futaba: TM7, TM8, TM10, TM14, etc.

FrSky: TF-8M, TF-14M

1.2 Specifications:

Model: TFR8

Number of channel: 8

Operating Voltage Range: 4V~10V

Dimension: 53mm*30mm*11mm

Weight: 13.5g

- Setup

2.1 Bind procedure:

Turn on the transmitter, connect the battery to the receiver while pressing receiver’s F/S button. After the RED LED is off and GREEN LED is solid, the binding process is completed and working mode is activated.

2.2 Setting failsafe:

TF series receivers support failsafe function for all channels.

After the receiver has been bind, press briefly the F/S button of the receiver, the GREEN LED of the receiver will flash twice, the failsafe is set up successfully.

If you do not need the failsafe function any more, just re-bind the receiver to default failsafe mode.

2.3 LED status:

| RED LED | GREEN LED | Mode |
| --- | --- | --- |
| Off | On | Working mode |
| On | On | Waiting to be bind |
| Flash | On | Signal lost |
| On | Flash twice | Set failsafe |

Should you have any questions or need further queries, pls send e-mails to us.

Technical support: frsky@frsky-rc.com or sales4tech@gmail.com.